导航传感器原理与应用

课程实验报告

| 实验名称： | 实验（2）：卫星导航实验 |
| --- | --- |
| 院（系）： | 未来工学院 |
| 姓 名： | 邓一骏 |
| 学 号： | 25SD36037 |
| 联系电话： | 18924314949 |
| 实验时间： | 2026年6月8日 |
| 实验成绩： |  |

空间控制与惯性技术研究中心制

一、实验目的

（1）深入理解GNSS定位的核心原理，掌握卫星位置迭代、接收机位置迭代的误差修正逻辑，理清卫星导航系统全链路工作机制；

（2）了解卫星伪距、卫星星历的数据结构，掌握从GNSS接收机输出数据中提取所需卫星导航系统伪距测量值、星历数据的方法；

（3）掌握利用“卫星位置+接收机位置”双环路迭代方法进行接收机定位解算的方法，完成GNSS单点定位程序编写，可自主设置迭代收敛阈值、异常终止规则并完成结果校验。

二、实验器材

（1）GNSS接收机1台；

（2）GNSS天线2个；

（3）数据采集及处理计算机1台。

三、实验内容

（1）完成GNSS定位系统线路连接；

（2）利用GNSS接收机配套软件完成单点定位，获取接收机天线所在位置的经度/纬度/高度信息、定位时刻UTC时间信息、定位时刻可见卫星编号/方位角/高度角信息；

（3）利用串口助手软件连接GNSS接收机，采集星历数据、伪距测量数据、卫星位置计算结果、单点定位计算结果；

（4）编写计算程序，利用星历数据及伪距测量数据完成接收机天线所在位置单点定位解算，并与（2）、（3）中采集的定位结果、可见卫星编号/方位角/高度角信息进行对比。

四、实验须知

（1）实验系统上电前检查线路连接是否正确，严禁线路短路；

（2）实验中禁止修改GNSS接收机串口波特率；

（3）GNSS定位系统冷启动需30s至60s，期间无定位结果输出；

（4）实验中可能使用到的GNSS接收机配置指令见下表：

| 编号 | 指令 | 说明 |
| --- | --- | --- |
| 1 | log satxyz2a once | 获取GNSS接收机计算的卫星位置信息 |
| 2 | log bestposa once | 获取GNSS接收机计算的单点定位结果 |
| 3 | log gpsephema once | 获取GNSS接收机接收到的星历数据 |
| 4 | log rangea once | 获取GNSS接收机测量的伪距数据 |

（5）GNSS接收机输出数据结构、指令构成详见实验指导材料。

五、原始数据及处理结果

（1）GNSS接收机配套软件界面截图/照片

（2）实验时可见卫星编号、用于定位卫星编号、定位授时结果

1. 全星座可见卫星统计：GPS: 8 颗；QZSS: 2 颗；GLONASS: 4 颗

BEIDOU: 12 颗，共计: 26 颗卫星

- GPS 卫星详情

| PRN | 状态 | 高度角 (°) | 方位角 (°) | ECEF-X (m) | ECEF-Y (m) | ECEF-Z (m) |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 参与定位 | 21.90 | 221.37 | -1725558.9643 | 26480396.7859 | -660070.0753 |
| 20 | 参与定位 | 47.26 | 203.63 | -10278878.4356 | 23655942.9564 | 6455394.0941 |
| 30 | 参与定位 | 27.79 | 100.67 | -24978896.4604 | -196654.5137 | 9600923.2127 |
| 5 | 参与定位 | 67.19 | 301.51 | -3348716.8234 | 15821607.5402 | 20885207.7424 |
| 6 | 参与定位 | 11.66 | 130.62 | -26104320.0957 | 4449812.6474 | -2895751.0123 |
| 7 | 参与定位 | 21.85 | 59.11 | -16916960.3871 | -9401533.6171 | 18981031.6988 |
| 21 | 参与定位 | 69.50 | 46.85 | -12799987.7546 | 8436316.6322 | 21693604.1427 |
| 11 | 参与定位 | 50.51 | 131.95 | -21484940.8617 | 12001655.3657 | 9939000.0173 |

3. 用于定位卫星 (GPS)

卫星编号 (PRN): [5, 6, 7, 9, 11, 15, 20, 21, 30]

卫星数量: 9 颗

4. 定位授时结果

GPS Week: 2421

t_obs (周内秒): 109502.000000 s

UTC 时间 (近似): 06:25:02.000

（3）原始星历数据

结果 (3): 原始星历数据 ===================== 格式: N×23 矩阵 列说明: [PRN, Week, A, Δn, M₀, e, ω, C_uc, C_us, C_rc, C_rs, C_ic, C_is, i₀, di/dt, Ω₀, Ω̇, t_oe, t_oc, T_GD, a_f0, a_f1, a_f2] Sat 1 (PRN 5): PRN: 5.000000e+00 Week: 2.421000e+03 A: 2.656115e+07 Δn: 3.939093e-09 M₀: 6.458152e-01 e: 5.603248e-03 ω: 1.444580e+00 C_uc: -5.630776e-06 C_us: 4.157424e-06 C_rc: 3.149688e+02 C_rs: -1.084062e+02 C_ic: -4.842877e-08 C_is: -1.359731e-07 i₀: 9.809644e-01 di/dt: -2.621538e-10 Ω₀: 2.437551e+00 Ω̇: -7.934616e-09 t_oe: 1.152000e+05 t_oc: 1.152000e+05 T_GD: -1.071021e-08 a_f0: -2.336310e-04 a_f1: -4.547470e-13 a_f2: 0.000000e+00 Sat 2 (PRN 6): PRN: 6.000000e+00 Week: 2.421000e+03 A: 2.656053e+07 Δn: 4.086599e-09 M₀: -1.649461e+00 e: 3.706335e-03 ω: -5.263274e-01 C_uc: 3.008172e-06 C_us: 6.724149e-07 C_rc: 3.811250e+02 C_rs: 5.753125e+01 C_ic: -5.587935e-08 C_is: -1.862645e-08 i₀: 9.858895e-01 di/dt: 7.714607e-11 Ω₀: 1.460755e+00 Ω̇: -8.156054e-09 t_oe: 1.152000e+05 t_oc: 1.152000e+05 T_GD: 3.725290e-09 a_f0: -5.969420e-04 a_f1: 4.774850e-12 a_f2: 0.000000e+00 Sat 3 (PRN 7): PRN: 7.000000e+00 Week: 2.421000e+03 A: 2.655949e+07 Δn: 5.021281e-09 M₀: -2.457602e+00 e: 2.087737e-02 ω: -1.964763e+00 C_uc: 5.401671e-08 C_us: 5.001202e-06 C_rc: 2.713438e+02 C_rs: 5.500000e+00 C_ic: 3.259629e-07 C_is: 3.725290e-09 i₀: 9.505527e-01 di/dt: 5.243076e-10 Ω₀: -1.708403e+00 Ω̇: -7.801396e-09 t_oe: 1.152000e+05 t_oc: 1.152000e+05 T_GD: -1.071021e-08 a_f0: -1.910020e-04 a_f1: -4.092730e-12 a_f2: 0.000000e+00 Sat 4 (PRN 9): PRN: 9.000000e+00 Week: 2.421000e+03 A: 2.656094e+07 Δn: 4.115529e-09 M₀: -1.260147e-01 e: 3.464701e-03 ω: 2.039375e+00 C_uc: -3.211200e-06 C_us: 1.005270e-05 C_rc: 1.872500e+02 C_rs: -5.756250e+01 C_ic: -1.117587e-08 C_is: -1.862645e-09 i₀: 9.661719e-01 di/dt: 5.189502e-10 Ω₀: -2.781045e+00 Ω̇: -7.695678e-09 t_oe: 1.080000e+05 t_oc: 1.080000e+05 T_GD: 4.656613e-10 a_f0: 7.430410e-04 a_f1: -5.002220e-12 a_f2: 0.000000e+00 Sat 5 (PRN 11): PRN: 1.100000e+01 Week: 2.421000e+03 A: 2.655980e+07 Δn: 4.641622e-09 M₀: -4.242671e-01 e: 2.312174e-03 ω: -2.356139e+00 C_uc: 2.501532e-06 C_us: 3.855675e-07 C_rc: 3.733750e+02 C_rs: 4.990625e+01 C_ic: -1.303852e-08 C_is: 6.146729e-08 i₀: 9.627861e-01 di/dt: -1.750073e-11 Ω₀: 1.477555e+00 Ω̇: -8.434280e-09 t_oe: 1.152000e+05 t_oc: 1.152000e+05 T_GD: -8.847564e-09 a_f0: -3.213800e-04 a_f1: 1.250560e-11 a_f2: 0.000000e+00 Sat 6 (PRN 15): PRN: 1.500000e+01 Week: 2.421000e+03 A: 2.656105e+07 Δn: 4.565547e-09 M₀: -6.865844e-01 e: 1.680077e-02 ω: 1.519968e+00 C_uc: -3.257766e-06 C_us: 9.948388e-06 C_rc: 1.837500e+02 C_rs: -6.381250e+01 C_ic: -1.844019e-07 C_is: 1.508743e-07 i₀: 9.442657e-01 di/dt: 4.496616e-10 Ω₀: -2.927240e+00 Ω̇: -8.189627e-09 t_oe: 1.152000e+05 t_oc: 1.152000e+05 T_GD: -1.071021e-08 a_f0: 4.164480e-04 a_f1: 3.183230e-12 a_f2: 0.000000e+00 Sat 7 (PRN 20): PRN: 2.000000e+01 Week: 2.421000e+03 A: 2.655969e+07 Δn: 4.150887e-09 M₀: 2.951598e+00 e: 2.036502e-03 ω: -1.823947e+00 C_uc: -3.157184e-06 C_us: 9.773299e-06 C_rc: 1.920312e+02 C_rs: -6.009375e+01 C_ic: 4.284084e-08 C_is: -1.303852e-08 i₀: 9.615528e-01 di/dt: 4.589477e-10 Ω₀: -2.775957e+00 Ω̇: -7.630675e-09 t_oe: 1.152000e+05 t_oc: 1.152000e+05 T_GD: -6.984919e-09 a_f0: 1.784540e-04 a_f1: 1.534770e-11 a_f2: 0.000000e+00 Sat 8 (PRN 21): PRN: 2.100000e+01 Week: 2.421000e+03 A: 2.655923e+07 Δn: 4.251249e-09 M₀: 2.624109e+00 e: 7.837947e-04 ω: -1.161341e-01 C_uc: -6.316230e-06 C_us: 4.354864e-06 C_rc: 2.987500e+02 C_rs: -1.214375e+02 C_ic: 8.381903e-08 C_is: -2.793968e-08 i₀: 9.639745e-01 di/dt: -2.164376e-10 Ω₀: 2.502019e+00 Ω̇: -8.036406e-09 t_oe: 1.152000e+05 t_oc: 1.152000e+05 T_GD: -9.778887e-09 a_f0: 4.119610e-04 a_f1: -2.160050e-12 a_f2: 0.000000e+00 Sat 9 (PRN 30): PRN: 3.000000e+01 Week: 2.421000e+03 A: 2.656064e+07 Δn: 5.434155e-09 M₀: -2.728740e+00 e: 8.242303e-03 ω: -2.268693e+00 C_uc: 9.313226e-08 C_us: 5.487353e-06 C_rc: 2.624062e+02 C_rs: 6.906250e+00 C_ic: 1.154840e-07 C_is: -1.396984e-07 i₀: 9.358425e-01 di/dt: 4.014453e-10 Ω₀: -1.718739e+00 Ω̇: -8.191413e-09 t_oe: 1.152000e+05 t_oc: 1.152000e+05 T_GD: 4.190952e-09 a_f0: 2.433130e-04 a_f1: 1.386980e-11 a_f2: 0.000000e+00

（4）原始伪距测量数据

结果 (4): 原始伪距测量数据 =========================== 格式: N×2 矩阵 PRN 伪距 (m) ------------------------- 5 20500355.213000 6 24787674.425000 7 24143180.745000 9 37833814.582000(不准，LSQ时需要剔除) 11 21411023.665000 15 23381976.522000 20 21506997.919000 21 20381448.455000 30 23121431.051000

（5）原始单点定位结果数据

结果 (5): 原始单点定位结果数据 =============================== BESTPOSA 参考结果 (接收机固件输出): 经度: 126.62859335° 纬度: 45.73246188° 高程: 141.1050 m 坐标系统: WGS84 定位模式: SINGLE (单点定位) GPS Week: 2421 周内秒: 109502.000 s

（6）原始卫星位置信息

结果 (6): 原始卫星位置信息 =========================== SATXYZ2A 固件输出的卫星 ECEF 坐标 (高度角/方位角基于哈尔滨计算): GPS PRN 15: 经度=93.728330°, 纬度=-1.424881°, 高度≈20166630.1m 高度角=21.90°, 方位角=221.37° GPS PRN 20: 经度=113.485703°, 纬度=14.051380°, 高度≈20210038.8m 高度角=47.23°, 方位角=203.62° GPS PRN 30: 经度=-179.548930°, 纬度=21.024238°, 高度≈20383060.1m 高度角=27.76°, 方位角=100.70° GPS PRN 5: 经度=101.950549°, 纬度=52.248190°, 高度≈20036424.7m 高度角=67.20°, 方位角=301.37° GPS PRN 6: 经度=170.326191°, 纬度=-6.240645°, 高度≈20260589.2m 高度角=11.67°, 方位角=130.62° GPS PRN 7: 经度=-150.937057°, 纬度=44.442765°, 高度≈20730015.8m 高度角=21.82°, 方位角=59.16° GPS PRN 21: 经度=146.611608°, 纬度=54.752556°, 高度≈20185439.3m 高度角=69.52°, 方位角=46.99° GPS PRN 11: 经度=150.811934°, 纬度=21.991982°, 高度≈20162890.4m 高度角=50.48°, 方位角=131.99° QZSS PRN 195: 经度=139.746784°, 纬度=27.389562°, 高度≈37981012.1m 高度角=65.66°, 方位角=145.69° QZSS PRN 199: 经度=127.038069°, 纬度=-0.044522°, 高度≈35775327.1m 高度角=37.53°, 方位角=179.43° GLONASS PRN 5+1: 经度=-147.887319°, 纬度=61.410470°, 高度≈19136971.5m 高度角=28.03°, 方位角=39.10° GLONASS PRN 20+2: 经度=149.204912°, 纬度=0.769596°, 高度≈19150458.7m 高度角=28.33°, 方位角=149.42° GLONASS PRN 21+4: 经度=129.228775°, 纬度=35.242386°, 高度≈19112346.0m 高度角=76.01°, 方位角=168.30° GLONASS PRN 7+5: 经度=92.557322°, 纬度=10.770200°, 高度≈19162468.2m 高度角=32.55°, 方位角=230.72° BEIDOU PRN 1: 经度=140.043228°, 纬度=0.193499°, 高度≈35769107.6m 高度角=36.12°, 方位角=161.46° BEIDOU PRN 8: 经度=116.277525°, 纬度=-1.717662°, 高度≈35595278.6m 高度角=34.70°, 方位角=193.95° BEIDOU PRN 6: 经度=99.963368°, 纬度=50.740505°, 高度≈35876103.2m 高度角=68.40°, 方位角=295.94° BEIDOU PRN 4: 经度=160.032339°, 纬度=2.111982°, 高度≈35770119.9m 高度角=30.12°, 方位角=136.00° BEIDOU PRN 3: 经度=110.528326°, 纬度=1.190302°, 高度≈35776708.1m 高度角=36.46°, 方位角=202.45° BEIDOU PRN 34: 经度=117.352013°, 纬度=22.234360°, 高度≈21526629.4m 高度角=58.65°, 方位角=201.06° BEIDOU PRN 22: 经度=-149.193462°, 纬度=47.991658°, 高度≈21527310.9m 高度角=23.17°, 方位角=54.71° BEIDOU PRN 21: 经度=137.999216°, 纬度=52.854466°, 高度≈21523944.2m 高度角=76.59°, 方位角=41.43° BEIDOU PRN 42: 经度=95.138881°, 纬度=24.773926°, 高度≈21516419.6m 高度角=48.57°, 方位角=241.31° BEIDOU PRN 2: 经度=80.000532°, 纬度=0.937272°, 高度≈35773609.1m 高度角=21.43°, 方位角=236.63° BEIDOU PRN 14: 经度=112.135619°, 纬度=40.296845°, 高度≈21520173.0m 高度角=74.71°, 方位角=248.61° BEIDOU PRN 31: 经度=144.692420°, 纬度=-13.784677°, 高度≈21546346.3m 高度角=15.81°, 方位角=159.96°

（7）单点定位程序数据信息（包含参与定位卫星编号/高度角/方位角/三维坐标、接收机经度/纬度/高度、与参考结果的偏差）

结果 (7): 单点定位程序数据信息 =============================== 参与定位卫星信息: PRN X_ECEF(m) Y_ECEF(m) Z_ECEF(m) 高度角(°) 方位角(°) ------------------------------------------------------------------------------------- 5 -3340770.9100 15828472.6693 20881298.3088 67.17 301.45 6 -26105585.1826 4450718.3243 -2883243.9474 11.69 130.61 7 -16925358.1230 -9398807.9301 18974830.9410 21.85 59.14 11 -21479967.2908 12001403.9141 9950135.8857 50.54 131.93 15 -1724384.0055 26480419.4339 -672383.4812 21.87 221.36 20 -10278731.8531 23659225.5153 6443478.8195 47.22 203.62 21 -12792567.6011 8444092.9522 21694940.9914 69.52 46.79 30 -24983043.7309 -194391.2822 9590033.2502 27.77 100.70 接收机位置 (自解算): 经度: 126.62921760° 纬度: 45.73234392° 高程: 209.3444 m ECEF: X=-2660879.8785, Y=3579063.3727, Z=4544681.7134 m 接收机钟差: 2.2020e-07 s 与参考结果的偏差: ΔE (东向): 48.5886 m ΔN (北向): -13.1536 m ΔU (天向): 68.2394 m 2D RMS (水平): 50.3375 m 3D Error: 84.7967 m 卫星位置验证 (与 SATXYZ2A 固件对比): PRN 15: 12369.358 m PRN 20: 12360.034 m PRN 30: 11870.693 m PRN 5: 11204.964 m PRN 6: 12603.467 m PRN 7: 10788.917 m PRN 21: 10831.297 m PRN 11: 12198.656 m

（8）完整的单点定位程序

""" GPS Single Point Positioning - Experiment 2 ============================================ Main entry point for the SPP pipeline. Pipeline: Data Extraction (A1-A3) → Satellite Position Computation (B) → Error Corrections (C1-C2) → Least Squares Solution (D) → Coordinate Transform (E1) → Accuracy Validation (E2) → Results Output & Visualization Design: 03_algorithm_design.md """ import os import sys import math import numpy as np # 将项目根目录加入 sys.path sys.path.insert(0, os.path.dirname(os.path.abspath(__file__))) import configs.constants as const from algorithms.extractor import ( extract_obs_time, extract_ephemeris_matrix, extract_pseudorange_matrix, ) from algorithms.satellite import compute_satellite_position from algorithms.corrections import ( apply_clock_correction, apply_earth_rotation_correction, ) from algorithms.solver import least_squares_solution from algorithms.transform import ecef_to_geodetic from evaluation.accuracy import validate_accuracy, parse_bestposa from visualization.plot_earth import ( parse_satxyz2a, plot_earth_and_satellites, compute_elev_azim ) def write_result(filepath, content): """写入结果文件""" os.makedirs(os.path.dirname(filepath), exist_ok=True) with open(filepath, 'w', encoding='utf-8') as f: f.write(content) print(f" -> Saved: {filepath}") def main(): print("=" * 60) print(" GPS Single Point Positioning - Experiment 2") print(" Design: 03_algorithm_design.md") print("=" * 60) # ---- Step 0: 读取数据 ---- data_path = const.DATA_FILE_PATH print(f"\n[0] Reading data: {data_path}") if not os.path.exists(data_path): print(f" ERROR: File not found: {data_path}") sys.exit(1) with open(data_path, 'r', encoding='utf-8') as f: raw_data = f.read() print(f" Source: {data_path} ({len(raw_data)} chars)") # ---- 解析 SATXYZ2A (用于可视化和卫星信息) ---- print("\n" + "-" * 50) print("[SATXYZ2A] Parsing satellite positions...") all_sats = parse_satxyz2a(raw_data) print(f" Total satellites: {len(all_sats)}") sys_count = {} for sat in all_sats: sys_name = sat[4] sys_count[sys_name] = sys_count.get(sys_name, 0) + 1 for s, c in sys_count.items(): print(f" {s}: {c}") has_reference = False # 初始化为False, 将在E2中更新 # ---- (A1): 提取观测时刻 ---- print("\n" + "-" * 50) print("[A1] Extracting observation time...") week, t_obs = extract_obs_time(raw_data) print(f" GPS Week: {week}, t_obs: {t_obs:.3f} s") # ---- (A2): 提取星历矩阵 ---- print("\n" + "-" * 50) print("[A2] Extracting GPS ephemeris matrix...") eph, valid_prns = extract_ephemeris_matrix(raw_data, week, t_obs) N = eph.shape[0] print(f" GPS satellites: {N}, PRNs: {valid_prns}") # ---- (A3): 提取伪距矩阵 ---- print("\n" + "-" * 50) print("[A3] Extracting pseudorange matrix...") pr = extract_pseudorange_matrix(raw_data, week, valid_prns) for i in range(N): print(f" PRN {int(pr[i, 0]):2d}: {pr[i, 1]:.3f} m") # ---- 一致性检查 ---- assert N == eph.shape[0] print(f"\n[5.5] Valid GPS satellites for positioning: {N}") if N < const.MIN_SATELLITES: print(f" ERROR: Insufficient satellites ({N} < " f"{const.MIN_SATELLITES})") sys.exit(1) # ---- (B): 卫星位置解算 ---- print("\n" + "-" * 50) print("[B] Computing satellite positions (ICD-GPS-200 9-step)...") sat_pos = np.zeros((N, 3), dtype=np.float64) ek_list = np.zeros(N, dtype=np.float64) e_list = np.zeros(N, dtype=np.float64) a_list = np.zeros(N, dtype=np.float64) sat_pos_info = [] # 用于结果输出: (PRN, X, Y, Z) — elev/azim 将在E2后补 for i in range(N): Xs, Ys, Zs, Ek, A, e = compute_satellite_position(eph[i], t_obs) sat_pos[i] = [Xs, Ys, Zs] ek_list[i] = Ek e_list[i] = e a_list[i] = A sat_pos_info.append((int(eph[i, 0]), Xs, Ys, Zs)) print(f" PRN {int(eph[i, 0]):2d}: " f"X={Xs:.2f}, Y={Ys:.2f}, Z={Zs:.2f} m") # ---- (C1): 卫星钟差修正 ---- print("\n" + "-" * 50) print("[C1] Applying clock correction...") rho_corr = np.zeros(N, dtype=np.float64) for i in range(N): dt_s = apply_clock_correction( eph[i], t_obs, ek_list[i], a_list[i], e_list[i] ) rho_corr[i] = pr[i, 1] + const.C * dt_s # 剔除异常卫星 PRN 9 (伪距 37,834 km 远超物理上限) valid_mask = [i for i in range(N) if int(eph[i, 0]) != 9] if len(valid_mask) < N: print(f" Excluding PRN 9 (anomalous pseudorange)") eph = eph[valid_mask] pr = pr[valid_mask] sat_pos = sat_pos[valid_mask] rho_corr = rho_corr[valid_mask] sat_pos_info = [sat_pos_info[i] for i in valid_mask] N = len(valid_mask) # ---- (C2): 地球自转修正 ---- print("\n" + "-" * 50) print("[C2] Applying Earth rotation correction...") sat_pos_corr = np.zeros_like(sat_pos) for i in range(N): Xc, Yc, Zc = apply_earth_rotation_correction( sat_pos[i, 0], sat_pos[i, 1], sat_pos[i, 2], rho_corr[i] ) sat_pos_corr[i] = [Xc, Yc, Zc] # ---- (D): 最小二乘迭代定位 ---- print("\n" + "-" * 50) print("[D] Least squares positioning...") X_rx = least_squares_solution(sat_pos_corr, rho_corr) clock_bias_s = X_rx[3] / const.C print(f" ECEF: X={X_rx[0]:.4f}, Y={X_rx[1]:.4f}, Z={X_rx[2]:.4f} m") print(f" Clock bias: {clock_bias_s:.4e} s") # ---- (E1): ECEF → 大地坐标 ---- print("\n" + "-" * 50) print("[E1] ECEF → Geodetic...") lon_s, lat_s, h_s = ecef_to_geodetic(X_rx[0], X_rx[1], X_rx[2]) print(f" Result: lon={lon_s:.8f} deg, " f"lat={lat_s:.8f} deg, h={h_s:.4f} m") # ---- (E2): 精度评估 ---- print("\n" + "-" * 50) print("[E2] Accuracy validation...") try: ref_lon, ref_lat, ref_h = parse_bestposa(raw_data) print(f" Reference: lon={ref_lon:.8f} deg, " f"lat={ref_lat:.8f} deg, h={ref_h:.4f} m") dE, dN, dU, r2d, r3d = validate_accuracy( lon_s, lat_s, h_s, ref_lon, ref_lat, ref_h ) has_reference = True except ValueError as e: print(f" WARNING: {e}") dE = dN = dU = r2d = r3d = 0.0 has_reference = False # 补全卫星高度角/方位角 (基于哈尔滨参考坐标) if has_reference: rx_lat_r = ref_lat * const.D2R rx_lon_r = ref_lon * const.D2R sl = math.sin(rx_lat_r) cl = math.cos(rx_lat_r) Nr = const.A_WGS84 / math.sqrt(1.0 - const.E2_WGS84 * sl * sl) rx_ecef_ref1 = ( (Nr + ref_h) * cl * math.cos(rx_lon_r), (Nr + ref_h) * cl * math.sin(rx_lon_r), (Nr * (1.0 - const.E2_WGS84) + ref_h) * sl ) sat_pos_info_aug = [] for item in sat_pos_info: prn, Xs, Ys, Zs = item elev_sat, azim_sat = compute_elev_azim((Xs, Ys, Zs), rx_ecef_ref1) sat_pos_info_aug.append((prn, Xs, Ys, Zs, elev_sat, azim_sat)) sat_pos_info = sat_pos_info_aug # ---- 卫星位置对比验证 (vs SATXYZ2A) ---- print("\n" + "-" * 50) print("[VERIFY] Computed GPS positions vs SATXYZ2A...") sat_verify_lines = [] for line in raw_data.split('\n'): if 'SATXYZ2A' not in line or ',GPS,' not in line: continue data_part = line.split(';')[1].split('*')[0] fields = data_part.split(',')[1:] for j in range(0, len(fields) - 6, 10): try: if fields[j] != 'GPS': continue prn_sat = int(fields[j + 1]) xs = float(fields[j + 2]) ys = float(fields[j + 3]) zs = float(fields[j + 4]) for i in range(N): if int(eph[i, 0]) == prn_sat: diff = math.sqrt( (sat_pos[i, 0] - xs) ** 2 + (sat_pos[i, 1] - ys) ** 2 + (sat_pos[i, 2] - zs) ** 2 ) line_str = f" PRN {prn_sat:2d}: diff={diff:.3f} m" print(line_str) sat_verify_lines.append(f"PRN {prn_sat:2d}: {diff:.3f} m") break except (ValueError, IndexError): continue break # ============================================================ # 结果输出到 results/ 目录 (对应实验报告模板 8 项) # ============================================================ print("\n" + "=" * 60) print(" Generating result files...") print("=" * 60) # ---- (1) 软件界面截图 — N/A (外部操作) ---- write_result( "exp2/results/01_software_screenshot.txt", "实验(2) 卫星导航实验\n" "====================\n\n" "结果 (1): GNSS接收机配套软件界面截图/照片\n" "说明: 请在此处粘贴 NovAtel Connect 软件界面的截图/照片。\n" "截图应显示定位状态、经纬高信息、UTC时间及可见卫星。\n" "参考数据:\n" f" BESTPOSA参考结果: lon={ref_lon:.8f}°, " f"lat={ref_lat:.8f}°, h={ref_h:.4f}m\n" f" 定位时刻: GPS Week={week}, t_obs={t_obs:.3f}s\n" ) # ---- (2) 可见卫星编号、用于定位卫星编号、定位授时结果 ---- all_visible_prns = sorted(set( int(sat[3]) for sat in all_sats if sat[4] == 'GPS' )) result2 = ( "结果 (2): 实验时可见卫星编号、用于定位卫星编号、定位授时结果\n" "=============================================================\n\n" "1. 全星座可见卫星统计:\n" ) for s, c in sys_count.items(): result2 += f" {s}: {c} 颗\n" result2 += f"\n 共计: {len(all_sats)} 颗卫星\n\n" result2 += "2. GPS 卫星详情:\n" result2 += " PRN 状态 高度角(°) 方位角(°) ECEF-X(m) " result2 += "ECEF-Y(m) ECEF-Z(m)\n" result2 += " " + "-" * 78 + "\n" used_gps_set = set(int(pr[i, 0]) for i in range(N)) # 构建 GPS 卫星 ECEF 查找表 gps_ecef = {} for line in raw_data.split('\n'): if 'SATXYZ2A' not in line or ',GPS,' not in line: continue dp = line.split(';')[1].split('*')[0] ff = dp.split(',')[1:] for k in range(0, len(ff) - 9, 10): try: if ff[k] == 'GPS': prn_gps = int(ff[k + 1]) gps_ecef[prn_gps] = ( float(ff[k + 2]), float(ff[k + 3]), float(ff[k + 4]) ) except (ValueError, IndexError): continue break # 计算接收机ECEF坐标(用于高度角/方位角计算) if has_reference: rx_lat_r = ref_lat * const.D2R rx_lon_r = ref_lon * const.D2R sl = math.sin(rx_lat_r) cl = math.cos(rx_lat_r) Nr = const.A_WGS84 / math.sqrt(1.0 - const.E2_WGS84 * sl * sl) rx_ecef_ref2 = ( (Nr + ref_h) * cl * math.cos(rx_lon_r), (Nr + ref_h) * cl * math.sin(rx_lon_r), (Nr * (1.0 - const.E2_WGS84) + ref_h) * sl ) for sat in all_sats: if sat[4] != 'GPS': continue prn = int(sat[3]) status = "参与定位" if prn in used_gps_set else "可见未用" # 计算高度角和方位角 xf, yf, zf = gps_ecef.get(prn, (0.0, 0.0, 0.0)) elev = 0.0 azim = 0.0 if has_reference and xf != 0.0: elev, azim = compute_elev_azim((xf, yf, zf), rx_ecef_ref2) result2 += f" {prn:2d} {status} {elev:6.2f} {azim:6.2f} " result2 += f"{xf:14.4f} {yf:14.4f} {zf:14.4f}\n" result2 += f"\n3. 用于定位卫星 (GPS): PRN {sorted(list(used_gps_set))}\n" result2 += f" 卫星数量: {len(used_gps_set)} 颗\n" result2 += f"\n4. 定位授时结果:\n" result2 += f" GPS Week: {week}\n" result2 += f" t_obs (周内秒): {t_obs:.6f} s\n" utc_hour = int((t_obs % 86400) / 3600) utc_min = int(((t_obs % 86400) % 3600) / 60) utc_sec = (t_obs % 86400) % 3600 % 60 result2 += f" UTC 时间 (近似): {utc_hour:02d}:{utc_min:02d}:{utc_sec:06.3f}\n" write_result("exp2/results/02_visible_satellites.txt", result2) # ---- (3) 原始星历数据 ---- result3 = "结果 (3): 原始星历数据\n" result3 += "=====================\n\n" result3 += "格式: N×23 矩阵\n\n" result3 += "列说明:\n" result3 += " [PRN, Week, A, Δn, M₀, e, ω, C_uc, C_us, C_rc, C_rs,\n" result3 += " C_ic, C_is, i₀, di/dt, Ω₀, Ω̇, t_oe, t_oc, T_GD, a_f0, a_f1, a_f2]\n\n" for i in range(N): result3 += f"Sat {i+1} (PRN {int(eph[i,0])}):\n" col_names = [ "PRN", "Week", "A", "Δn", "M₀", "e", "ω", "C_uc", "C_us", "C_rc", "C_rs", "C_ic", "C_is", "i₀", "di/dt", "Ω₀", "Ω̇", "t_oe", "t_oc", "T_GD", "a_f0", "a_f1", "a_f2" ] for c in range(23): result3 += f" {col_names[c]}: {eph[i,c]:.6e}\n" result3 += "\n" write_result("exp2/results/03_ephemeris_matrix.txt", result3) # ---- (4) 原始伪距测量数据 ---- result4 = "结果 (4): 原始伪距测量数据\n" result4 += "===========================\n\n" result4 += "格式: N×2 矩阵\n\n" result4 += " PRN 伪距 (m)\n" result4 += " " + "-" * 25 + "\n" for i in range(N): result4 += f" {int(pr[i,0]):3d} {pr[i,1]:.6f}\n" write_result("exp2/results/04_pseudorange.txt", result4) # ---- (5) 原始单点定位结果数据 ---- result5 = "结果 (5): 原始单点定位结果数据\n" result5 += "===============================\n\n" result5 += "BESTPOSA 参考结果 (接收机固件输出):\n" result5 += f" 经度: {ref_lon:.8f}°\n" result5 += f" 纬度: {ref_lat:.8f}°\n" result5 += f" 高程: {ref_h:.4f} m\n" result5 += f" 坐标系统: WGS84\n" result5 += f" 定位模式: SINGLE (单点定位)\n" result5 += f" GPS Week: {week}\n" result5 += f" 周内秒: {t_obs:.3f} s\n" write_result("exp2/results/05_bestposa_reference.txt", result5) # ---- (6) 原始卫星位置信息 ---- result6 = "结果 (6): 原始卫星位置信息\n" result6 += "===========================\n\n" result6 += "SATXYZ2A 固件输出的卫星 ECEF 坐标 (高度角/方位角基于哈尔滨计算):\n\n" for sat in all_sats: lon, lat, h, prn, sys_name = sat[:5] # 计算ECEF坐标 sl = math.sin(lat * const.D2R) cl = math.cos(lat * const.D2R) N_sat = const.A_WGS84 / math.sqrt(1.0 - const.E2_WGS84 * sl * sl) sv_ecef = ( (N_sat + max(h, 0)) * cl * math.cos(lon * const.D2R), (N_sat + max(h, 0)) * cl * math.sin(lon * const.D2R), (N_sat * (1.0 - const.E2_WGS84) + max(h, 0)) * sl ) elev, azim = 0.0, 0.0 if has_reference: elev, azim = compute_elev_azim(sv_ecef, rx_ecef_ref2) result6 += f" {sys_name} PRN {prn}:\n" result6 += f" 经度={lon:.6f}°, 纬度={lat:.6f}°, 高度≈{h:.1f}m\n" result6 += f" 高度角={elev:.2f}°, 方位角={azim:.2f}°\n\n" write_result("exp2/results/06_satellite_positions.txt", result6) # ---- (7) 单点定位程序数据信息 ---- result7 = "结果 (7): 单点定位程序数据信息\n" result7 += "===============================\n\n" result7 += "参与定位卫星信息:\n" result7 += " PRN X_ECEF(m) Y_ECEF(m) Z_ECEF(m) " result7 += "高度角(°) 方位角(°)\n" result7 += " " + "-" * 85 + "\n" for item in sat_pos_info: if len(item) == 6: prn, Xs, Ys, Zs, elev_sat, azim_sat = item else: prn, Xs, Ys, Zs = item elev_sat, azim_sat = 0.0, 0.0 result7 += f" {prn:3d} {Xs:16.4f} {Ys:16.4f} {Zs:16.4f} " result7 += f"{elev_sat:8.2f} {azim_sat:8.2f}\n" result7 += f"\n接收机位置 (自解算):\n" result7 += f" 经度: {lon_s:.8f}°\n" result7 += f" 纬度: {lat_s:.8f}°\n" result7 += f" 高程: {h_s:.4f} m\n" result7 += f" ECEF: X={X_rx[0]:.4f}, Y={X_rx[1]:.4f}, " result7 += f"Z={X_rx[2]:.4f} m\n" result7 += f" 接收机钟差: {clock_bias_s:.4e} s\n" if has_reference: result7 += f"\n与参考结果的偏差:\n" result7 += f" ΔE (东向): {dE:.4f} m\n" result7 += f" ΔN (北向): {dN:.4f} m\n" result7 += f" ΔU (天向): {dU:.4f} m\n" result7 += f" 2D RMS (水平): {r2d:.4f} m\n" result7 += f" 3D Error: {r3d:.4f} m\n" result7 += f"\n卫星位置验证 (与 SATXYZ2A 固件对比):\n" for line in sat_verify_lines: result7 += f" {line}\n" write_result("exp2/results/07_positioning_results.txt", result7) # ---- (8) 程序说明 ---- result8 = "结果 (8): 完整的单点定位程序\n" result8 += "==============================\n\n" result8 += "程序组成:\n" result8 += " main.py - 主入口 (Pipeline控制器)\n" result8 += " configs/constants.py - 物理常数与算法超参数\n" result8 += " algorithms/extractor.py - A1:观测时刻 A2:星历矩阵 A3:伪距矩阵\n" result8 += " algorithms/satellite.py - B:卫星位置解算 (ICD 9步)\n" result8 += " algorithms/corrections.py - C1:钟差修正 C2:地球自转修正\n" result8 += " algorithms/solver.py - D:最小二乘迭代求解\n" result8 += " algorithms/transform.py - E1:ECEF→大地坐标\n" result8 += " evaluation/accuracy.py - E2:精度验证\n" result8 += " visualization/plot_earth.py - 可视化\n\n" result8 += f"输入数据: {data_path}\n" result8 += f"输出目录: exp2/results/\n\n" result8 += "运行方式:\n" result8 += " python exp2/main.py\n" write_result("exp2/results/08_program_info.txt", result8) # ============================================================ # 可视化 # ============================================================ print("\n" + "-" * 50) print("[VIZ] Generating visualization...") print(f" Showing {len(all_sats)} satellites") used_gps_prns = set(int(pr[i, 0]) for i in range(N)) # 使用哈尔滨参考坐标 (BESTPOSA) 作为天空图观测点 rx_ref = (ref_lon, ref_lat, ref_h) if has_reference else (lon_s, lat_s, h_s) # 构建用于计算高度角/方位角的卫星ECEF坐标矩阵(所有星座) sat_pos_all = np.zeros((len(all_sats), 3), dtype=np.float64) for i, sat in enumerate(all_sats): lon, lat, h, prn, sys_name = sat[:5] lat_r = lat * const.D2R lon_r = lon * const.D2R sl = math.sin(lat_r) N_sat = const.A_WGS84 / math.sqrt(1.0 - const.E2_WGS84 * sl * sl) sat_pos_all[i, 0] = (N_sat + max(h, 0)) * math.cos(lat_r) * math.cos(lon_r) sat_pos_all[i, 1] = (N_sat + max(h, 0)) * math.cos(lat_r) * math.sin(lon_r) sat_pos_all[i, 2] = (N_sat * (1.0 - const.E2_WGS84) + max(h, 0)) * sl plot_earth_and_satellites( all_sats, rx_ref, used_gps_prns=used_gps_prns, sat_pos_ecef=sat_pos_all, title=f"Multi-constellation SVs (t_obs={t_obs:.0f}s, " f"{len(all_sats)} SVs, {N} GPS used)", save_path="exp2/results/visualization.png" ) print("\n" + "=" * 60) print(" ALL 8 RESULTS GENERATED IN exp2/results/") print("=" * 60) if __name__ == '__main__': main()